Documentation for filled meteorology data

Contact:

Dr. Daniel Ricciuto

Environmental Science Division

Oak Ridge National Laboratory

Oak Ridge, TN 37831

ricciutodm@ornl.gov

Updates:

Click HERE for a log of updates to this file and the filled meteorology files.

Data format:

Files provided are in the ALMA netcdf standard.  The time resolution of these files matches that provided in the AmeriFlux level 2 data, which is half-hourly for most sites but hourly for several (e.g. Park Falls, Harvard Forest).  Time in each file is local standard (LST).  Filenames follow the tower coding conventions used in AmeriFlux level 4 data.

Filling methodology:

We use climate station data when available to fill gaps in tower meteorological data.  NCDC climate stations within 50km are available for all sites.  About half of these sites had hourly measurements, generally from ASOS sites.  The rest were usually coop sites.  In addition, DAYMET modeled fine-scale climate data are available for continental US sites through the year 2003.  When station data are not available, a 10-day running mean diurnal cycle is used.  The following list provides links to NCDC stations that were used to fill each site.  Detailed descriptions for individual variables follow below.

Temperature (Tair)

If hourly data from a nearby NCDC station are available, these are used to perform the filling.  Biases between the NCDC station and the tower are corrected by adding the difference in mean annual temperature

If daily data are available but not hourly, we use DAYMET or NCDC maximum and minimum temperature data.  If both options are available, we choose the option that has the best linear correlation with available tower data.  Biases are corrected by adding the difference in mean annual temperature between the tower and the climate data (separately for maximum and minimum temperature).  Daily data are converted to hourly (or half-hourly) using a sine wave assuming the following:  High temperature occurs at 15 LST, low temperature occurs at 3 LST.  Additionally, a 6-hr running mean filter is applied on filled data between 18 LST and 06 LST to prevent large discontinuities in temperature across day boundaries.

If neither hourly nor daily climate data are available, fill with diurnal mean method described above.

Near surface specific humidity (Qair)

If hourly T and RH data from a nearby NCDC station are available, convert these to Qair and use to fill.  Potential biases between the tower and NCDC station are not corrected.

If hourly climate data are not available, fill with diurnal mean method described above.

Wind speed (Wind)

If hourly wind speed data from a nearby NCDC station are available, use these to fill.  Potential biases between the tower and NCDC station are not corrected.

If hourly climate data are not available, fill with diurnal mean method described above.

Precipitation (Rainf)

If hourly precipitation data from a nearby NCDC station are available, use these to fill.  Potential biases between the tower and NCDC station are not corrected.

If daily data are available and not hourly, use DAYMET or NCDC precipitation data.  If both options are available, choose the option that has the best linear correlation with available tower data.  Potential biases between the tower and NCDC station are not corrected.  Daily data are converted to hourly (or half-hourly) by dividing the daily sum by 24 (or 48) and evenly distributing rainfall throughout the day.  We might want to distribute the precipitation only at night following the LBAmip procedure, suggestions are welcome.  Bai Yang and CDIAC are also working on a stochastic weather generator to fill precipitation gaps.

Surface pressure (Psurf)

If hourly surface pressure data from a nearby NCDC station are available, use these to fill.  Potential biases between the tower and NCDC station are not corrected.

If hourly data are not available, fill using diurnal mean method described above.

If pressure is not measured at all (occurs for several sites), fill using hydrostatic balance given site elevation.

Downwelling shortwave radiation (SWdown)

SWdown may be converted from PAR if PAR measurements are deemed to be more reliable.

If hourly cloud cover data from a nearby NCDC station are available, there are four categories:  0 (clear), 1 (scattered clouds), 2 (broken clouds) and 3 (overcast).  The existing tower SWdown measurements are divided into these four categories.  Diurnal mean averages are calculated for each day of the year for each cloud cover category.  Gaps are then filled using this diurnal information combined with the NCDC cloud cover data.

**NOTE:  as of 5/19/2008 the above method is currently disabled as it causes large biases for unknown reasons **

If DAYMET is available but not hourly NCDC, use DAYMET estimated radiation data.  DAYMET radiation is estimated from a simple model based on daily temperature range.  DAYMET daily radiation is downscaled to hourly (or half-hourly) values using a mean diurnal cycle.

If neither of the above is available, fill using diurnal mean method described above.

Downwelling longwave radiation (LWdown)

Emissivities were estimated using Bondville flux tower data based on temperature, humidity and cloud cover from a nearby NCDC station.  A lookup table of emissivities for a given temperature and cloud cover was created for use in gap-filling all sites.

If hourly cloud cover data from a nearby NCDC station are available, there are four categories:  0 (clear), 1 (scattered clouds), 2 (broken clouds) and 3 (overcast).  This information was used in combination with temperature and humidity measurements to look up an emissivity.  The emissivity was then used to calculate LWdown with air temperature using the Stefan-Boltzmann Law.  If hourly cloud cover were not available, another version of the lookup table was used that uses an average over sky conditions.

Carbon dioxide concentration (CO2air)

No tower measurements were used to produce CO2 concentrations.  Instead, an algorithm selects the closest continental flask site out of the following list:

Argyle, ME

Barrow, AK

Estevan point, BC

Park Falls, WI

La Jolla, CA

Niwot Ridge, CO

Moody, TX

Grifton, NC

Globalview CO2 data from these sites are then used.  No interpolation is performed.

NCDC stations used in filling

Return to top

Tower codes and additional information

Return to top

Netcdf file description

int time(time) ;

time:long_name = "Time axis" ;

time:units = "seconds since YYYY-01-01 00:00:00" ;

time:calendar = "gregorian" ;

time:time_origin = "YYYY-01-01 00:00:00" ;

int timestp(time) ;

timestp:long_name = "Time step axis" ;

timestp:units = "timesteps since YYYY-01-01 00:00:00" ;

timestp:tstep_sec = 3600. ;

timestp:time_origin = "YYYY-01-01 00:00:00" ;

double lon(x) ;

lon:long_name = "longitude" ;

lon:units = "degrees_east" ;

double lat(y) ;

lat:long_name = "latitude" ;

lat:units = "degrees_north" ;

double level(z) ;

level:long_name = "Vertical levels" ;

level:units = "m" ;

double Tair(x, y, z, time) ;

Tair:axis = "XYZT" ;

Tair:long_name = "Near surface air temperature" ;

Tair:units = "K" ;

Tair:missing_value = -9999. ;

double Qair(x, y, z, time) ;

Qair:axis = "XYZT" ;

Qair:long_name = "Near surface specific humidity" ;

Qair:units = "kg/kg" ;

Qair:missing_value = -9999. ;

double Wind(x, y, z, time) ;

Wind:axis = "XYZT" ;

Wind:long_name = "Near surface module of the wind" ;

Wind:units = "m/s" ;

Wind:missing_value = -9999. ;

double Rainf(x, y, z, time) ;

Rainf:axis = "XYZT" ;

Rainf:long_name = "Rainfall rate" ;

Rainf:units = "kg/m2/s" ;

Rainf:missing_value = -9999. ;

double Psurf(x, y, z, time) ;

Psurf:axis = "XYZT" ;

Psurf:long_name = "Surface pressure" ;

Psurf:units = "Pa" ;

Psurf:missing_value = -9999. ;

double SWdown(x, y, z, time) ;

SWdown:axis = "XYZT" ;

SWdown:long_name = "Surface incident shortwave radiation" ;

SWdown:units = "W/m2" ;

SWdown:missing_value = -9999. ;

double LWdown(x, y, z, time) ;

LWdown:axis = "XYZT" ;

LWdown:long_name = "Surface incident longwave radiation" ;

LWdown:units = "W/m2" ;

LWdown:missing_value = -9999. ;

double CO2air(x, y, z, time) ;

CO2air:axis = "XYZT" ;

CO2air:long_name = "Near surface CO2 concentration" ;

CO2air:units = "ppmv" ;

CO2air:missing_value = -9999. ;

Return to top

Updates

May 13th, 2008:  Uploaded 30 US sites

Note:  please do not use the .tar file provided in ftp://nacp.ornl.gov/synthesis/2008/upload.  This was for testing purposes only.

Return to top